Pre-Flight Check

Park brake ………………………..……………………….. ON Merlin……………………………………………….... Running Battery Master Switch ..ON Fuel Quantity ……………….... Checked and Verified Avionics …………………………………..…..…………….. ON Record airport information Avionics ……………………………….…………….……... OFF Flight Controls …………..… FULL, FREE & CORRECT Flaps…….………………. Extend/Retract FULL CHECK Battery Master Switch………………………………...OFF Briefing ………………………………..….…….. Completed

Pre-Start Check

Park Brake ON Hatches & Harnesses Secure Elevator Trim Op Checked and Set Circuit Breakers Check All IN Avionics Main S/W OFF Fuel Tank Selector BOTH Fuel Valve …………………………………………..……. ON Throttle ………………………………….. OPEN 1/2 inch Mixture Idle Cut-Off Propeller Area Clear Master Switches (both) ON Beacon & Nav Lights ON Fuel Pump ON Mixture Full Rich Pause, and register fuel flow rise Fuel Pump OFF Magneto/Start S/W START Check a/c stationary, set 1,500 rpm for 3 mins, then set to 1,200 rpm Avionics Master ON Radios ON & tune as req’d Taxi Lights …………….…………........................... ON

Taxy Check

Taxy area ……………………….………. Check Clear Toe brakes …………………….……… Check BOTH Flight Instruments ……………….…………. Check

Engine Check

Park brake …………………………………..……… ON Fuel Tank selector ……………………..…… BOTH Mixture …………………………………… FULL RICH Throttle ……………………………… Set 1,800 rpm Check a/c stationary Engine instruments ………………………… Check Magnetos/Plugs ………….……. Check (150/50) Throttle ….. Check IDLE, then 1,200 rpm SET

Before Take-Off Check

Flight Controls Full & Free Flight Instruments Set Fuel Tank Selector BOTH Hatches & harnesses Secure Mixture Full Rich Fuel Pump ………………………………..……………… ON Flaps As Req’d Elevator Trim Set Radios Set ----------------- Altimeter ……………………….……………….. QNH SET Transponder C mode ON Pitot Heat …………………….…………….……………. ON Lights ……………….……..…… Landing & Strobe ON Approach ……..…………………………….. Check Clear

After Take-Off Check

Flaps . Retract on Schedule Engine Instruments ………….…….. T’s & P’s CHECK Lights ………………………..………………………. As Req’d Fuel Pump ………………………………………..………. OFF

Approach Checks (BUMFFITCH-L)

Brakes ………………………..………. Checked and OFF Undercarriage DOWN & Locked Mixture Full Rich Fuel .… TANKS to BOTH/ CHECK QTY/PUMP ON Flaps Extend on Schedule Instruments Check Temps & Press’s .Check Carb Heat ..Check (not fitted) Hatches and Harnesses ..Secure Landing & Taxi Lights ………………………………….. ON

Landing Check

Flaps …………………………………….…….. Landing SET

After-Landing Check

Flaps Retract Transponder, Landing & Strobe Lights ……. OFF Pitot Heat and Probe Heat ……………….…….. OFF

Shut-Down Check

Park Brake ON Throttle Set 1,200 rpm Avionics Main S/W & Transponder OFF Magnetos & Plugs Check (L-R-OFF-BOTH) Mixture Idle Cut-Off Remaining equipment & lights OFF Magnetos OFF Master Switches (both) OFF Merlin ……………………………… Confirm PIREP Sent

Engine Failure (During Flight)

Airspeed …………………………………………… 68 KIAS Landing Site ……………………..……………. CHOOSE Fuel ………………………….……. SELECTOR to BOTH Mixture ……………………………………….. FULL RICH Fuel Pump ………………………………………………. ON Fuel Quantity ……………………………….…….. CHECK Magnetos ………………….……. CHECK (L/R/BOTH) Radios ………………..……. DECLARE EMERGENCY Transponder …………………………………… SET 7700 No Restart? ………………. EMERGENCY LANDING

Emergency Landing (without engine power)

Airspeed ………….…………………70 KIAS (FLAPS UP) 65 KIAS (FLAPS DOWN) Fuel Shutoff Valve …………………………………… OFF Mixture ……………………………………. IDLE CUT-OFF Ignition Switch ………………………………………… OFF Flaps ……………. AS REQD (30° RECOMMENDED) When landing assured: Master Switches (both) ……………………………. OFF Doors ……………………………………………UNLATCHED Touchdown ………………………..………….…TAIL LOW Brakes …………………………………………………. APPLY

Reference Speeds..

Rotation ……………………………………….... 55-60 KIAS Normal Climb Out …………………………... 75-85 KIAS Short Field Takeoff ……………………….…. Vr: 51 KIAS Maintain 56 KIAS until clear of obstacles VY (Best Rate of Climb), Sea Level …….……74 KIAS VY (Best Rate of Climb 10,000ft) ……………. 72 KIAS VX (Best Angle of Climb Sea Level)…….……. 62 KIAS VX (Best Angle of Climb) 10,000ft …………... 67 KIAS Normal Approach, Flaps UP……………….. 65-75 KIAS Normal Approach, Flaps FULL ……..….... 60-70 KIAS Short Field Approach, Flaps FULL …………... 61 KIAS Balked Landing …………………………..……..…… 60 KIAS Maximum Power, Flaps 20 Turbulent Air Penetration Speeds 2550lb …………………………………………….……. 105 KIAS 2200lb ……………………………………………………. 98 KIAS 1900lb ……………………………………………………. 90 KIAS Maximum Glide …………………………………….. 68 KIAS.

HASELL (Prior to initiating Stalls/Aerobatics)

Height …………. SUFFICIENT (See Flying Orders) Airframe …………………………. SET FLAP AS REQD Security ……… HARNESSES & HATCHES SECURE NO LOOSE ARTICLES Engine …………………………… MIXTURE FULL RICH CHECK INDICATIONS Location ……………………CLEAR OF A, B, C and D: Active Airfields, Built-up Areas, Cloud, Controlled Airspace and Danger Areas Lookout …………………………………………. COMPLETE 2 x 90° turns L and R, or 1 x 180° turn

HELL (between subsequent manoeuvres)

Height ……………………..………………… SUFFICIENT Engine …………………..……… MIXTURE FULL RICH CHECK INDICATIONS Location …………………….. CLEAR OF A, B, C and D: Active Airfields, Built-up Areas, Cloud, Controlled Airspace and Danger Areas Lookout …………………………………………. COMPLETE 1 x 90° turn

Limitations

Max Flap Speed 100 Flap …………….…. 110 KIAS Max Flap Speed 200 to Full Flap…… ..… 85 KIAS Max Demonstrated Crosswind ……….….15 KIAS Max Wind Velocity (Taxi) …………….……. 27 KIAS If taxiway contaminated: 5 KIAS Max Headwind ………………….…………..……27 KIAS Max Tailwind ………………..……………..……. 10 KIAS Max TO Weight ....... 2550 LB (Normal Category) 2200 LB (Utility Category) Maximum Weight in Baggage Compartment: Normal Category: Baggage Area 1: 120 lb Baggage Area 2: 50 lb Total combined weight must not exceed 120 lb Utility Category: Rear seat must not be occupied Baggage compartment must be empty Maximum Bank Angle … 60° (Normal Category) Max Weight for Spins ………………….……… 2100 lb Max Number of Occupants …….… PILOT + 3 PAX Max Operating Altitude …………..………….. 9,800 ft OAT Limits ………………………………..... -40°C to +40°C

FREDA

Fuel ……………………………… CHECK SUFFICIENT Radios ……………………………. SET AND CHECKED Engine …………………………. CHECK INDICATIONS DI ………………………………ALIGN WITH COMPASS Altimeter …………. SET & CHECKED (QNH or QFE)